Pre-normative research for safety of hydrogen driven vehicles and transport through tunnels and similar confined spaces

Fuel Cells and Hydrogen Joint Undertaking (FCH JU)

Grant Agreement Number 826193

| Field | Possible information |
| --- | --- |
| Summary | WP 2 Unignited leaks in tunnels and confined space 2.4.2 Unignited pressure peaking phenomenon Unignited pressure peaking phenomenon results are based on an experimental series from spring 2019 in Horten, Norway. |
| Data file name | Concentration data (Excel worksheet):; Pressure data (txt document):; Mass flow rate data (folder):. HTE242USN00002CONC190621; HTE242USN00002PRESS190621; HTE242USN00001MFR190621. HTE242USN00003CON190621; HTE242USN00003PRESS190621; HTE242USN00002MFR190621. HTE242USN00006CON190621; HTE242USN00004PRESS190621; HTE242USN00003MFR190621. HTE242USN00007CON190621; HTE242USN00005PRESS190621; HTE242USN00004MFR190621. HTE242USN00008CON190621; HTE242USN00006PRESS190621; HTE242USN00005MFR190621. HTE242USN00009CONC190621; HTE242USN00007PRESS190621; HTE242USN00006MFR190621. HTE242USN00009CONC190621; HTE242USN00008PRESS190621; HTE242USN00007MFR190621. HTE242USN00010CONC190621; HTE242USN00009PRESS190621; HTE242USN00008MFR190621. HTE242USN00010CONT190621; HTE242USN00010PRESS190621; HTE242USN00009MFR190621. HTE242USN00011CONT190621; HTE242USN00011PRESS190621; HTE242USN00010MFR190621. |
| Author | André Vagner Gaathaug and Agnieszka W. Lach 17.06.2019 to 21.06.2019 Horten, Norway André Vagner Gaathaug |
| Experimental setup | HyTUNNEL-CS D2.2 Intermediate report on analytical, numerical and experimental studies Figur 1: Sketch of the experimental setup Measurement devices: Concentration data files: H1 and H2 -> XEN-352 sensors Pressure data files: P1 [kPa] -> XTM-190-10A Mass flow data files: Mass Flow [V] ->Micro Motion ELITE & Pressure [V] at mass flow meter -> ASI |
| Objective of the experiment | Experiment goals - Provide experimental evidence of the Pressure Peaking Phenomenon in large scale. - Provide validation data for CFD and engineering models. |
| Experimental procedure | Exp nr; Nozzle diameter [m]; Enclosure volume [m3]; Vent area [m2]; Vent size [mm]; Mass flow rate [g/s]; Static pressure -reservoir [MPa]; Experimental overpressure [kPa]; Exp time for pressure peak [s]; Duration time [s]. 2; 0.004; 14.9; 0.0012; 9x 13; 1.9; 2.68; 0.42; 10; 90. 3; 0.004; 14.9; 0.002; 1x 51,6; 3.5; 4.0; 0.51; 10; 120. 4; 0.004; 14.9; 0.002; 1x 51,6; 9.05; 10.4; 2.86; 17; 120. 5; 0.004; 14.9; 0.0014; 1x 42; 9.9; 11.0; 6.45; 37; 120. 6; 0.004; 14.9; 0.0014; 1x 42; 10.1; 11.75; 6.74; 37; 120. 7; 0.004; 14.9; 0.0006 *; 4x 13; 3.05; 3.6; 4.07; 80; 180. 8; 0.004; 14.9; 0.0006*; 4x 13; 3.05; 3.97; 3.96; 77; 180. 9; 0.004; 14.9; 0.0006*; 4x 13; 4.75; 5.85; 8.05; 89; 200. 10; 0.004; 14.9; 0.0006*; 4x 13; 4.2; 5.26; 6.70; 89; 200. 11; 0.004; 14.9; 0.0006*; 4x 13; blowdown; 4.96; 7.00; 64; 1000. |
| Experimental data | Description - The experimental data is provided in three separate parts. - Concentration data from 8 experiments are given in a excel datasheet format with the second and third column as the time and concentration data. - The pressure data is given as text format (ASCII) Time P1 Trigger s kPa V - The mass flow data is given in text format in the corresponding folder, where CH1 is the time vector, CH2 is the mass flow rate given in Volts. It converts to g/s by mfr = (U-1)*6.25 and CH3 is the pressure at the mass flow meter given in units Volts. It converts to unit bar by p=62.5*(U-1). Experiment data - The figure shows an example of the measured pressure against time. The data is properly scaled and shows the overpressure in unit kPa and time in seconds. Figure 1: Experiment 11 |
| Comments | No changes made No validation made |
| Reference | From the participants or a third party (if it is involved) the reports and papers published including: Paper: https://doi.org/10.1016/j.ijhydene.2020.08.221 - HyTunnel-CS D2.2 Intermediate report on analytical, numerical and experimental studies |
| Acknowledgements | The data presented is part of the experimental results of HyTunnel-CS The project HyTunnel-CS has received funding from the Fuel Cells and Hydrogen 2 Joint Undertaking (JU) under grant agreement No 826193. The JU receives support from the European Union’s Horizon 2020 research and innovation programme and United Kingdom, Germany, Greece, Denmark, Spain, Italy, Netherlands, Belgium, France, Norway, Switzerland |